SHORT COURSE SYLLABUS

Basic Robotics &

Its Programming

A Hands-On Beginner Course with Arduino

| DURATION 8 Hours | MODULES 4 Sessions | HARDWARE Arduino Uno | LEVEL Beginner |
| --- | --- | --- | --- |

| Institution |  |
| --- | --- |
| Instructor | Prof. Dr. Syed Ahmed |
| Course Code |  |
| Academic Term | 2026 |

# 1. Course Overview

Basic Robotics and Its Programming is an intensive 8-hour beginner course that teaches the fundamental concepts of robotics through hands-on Arduino programming. Students start with zero experience and finish with a working, sensor-driven autonomous robot they built and programmed themselves.

Every concept is introduced through building — not slides. Students wire circuits, upload code, observe results, and iterate. The course is structured into four 2-hour modules that follow a clear progression: understand the robot, sense the world, make it move, then make it think.

What Makes This Course Different 8 hours is enough to go from zero to a functioning autonomous robot. No prior coding or electronics knowledge required — just curiosity. Every session produces a working output: a blinking LED, a reacting sensor, a moving motor. The final module is a live capstone demo where students show their robot to the class.

# 2. Course Objectives

By the end of this course, students will be able to:

- Describe what a robot is and identify its three core components

- Set up the Arduino IDE and upload programs to an Arduino Uno

- Write Arduino code using variables, functions, and conditional logic

- Wire sensors (ultrasonic, LDR) and read their data in Serial Monitor

- Control a servo motor to precise angles using the Servo library

- Drive a DC motor in both directions at variable speeds

- Program a mobile robot to navigate and avoid obstacles autonomously

- Debug hardware and software problems using a systematic approach

- Complete and demonstrate a capstone robot project to peers

# 3. Target Audience & Prerequisites

## Who This Course Is For

This course is designed for complete beginners with no background in electronics, programming, or robotics:

- Secondary school students (ages 12 and above)

- University students exploring engineering or computer science

- Adult learners entering the technology sector

- Hobbyists and makers taking their first steps with Arduino

- Teachers seeking a structured robotics workshop curriculum

## Prerequisites

No technical knowledge required Basic computer literacy: comfortable using a mouse, keyboard, and installing software. Ability to follow step-by-step written and verbal instructions. Willingness to experiment — errors and re-tries are a core part of learning.

# 4. Required Materials & Tools

## Hardware (per student or pair)

| Component | Qty | Purpose |
| --- | --- | --- |
| Arduino Uno R3 | 1 | Core microcontroller |
| USB Type-B cable | 1 | PC connection |
| Breadboard (830 points) | 1 | Prototyping |
| Jumper wires (M-M) | 30+ | Various lengths |
| HC-SR04 ultrasonic sensor | 1 | Distance sensing |
| LDR photoresistor | 1 | Light sensing |
| 10k ohm resistor | 3 | Pull-down & voltage divider |
| SG90 servo motor | 1 | Precise angle control |
| L298N motor driver | 1 | DC motor control |
| DC hobby motors 3-6V | 2 | Wheel drive |
| LEDs (assorted) | 5 | Visual feedback |
| 220 ohm resistors | 5 | LED current limiting |
| Potentiometer 10k | 1 | Servo angle input |
| 9V battery + connector | 1 | Motor power |
| Robot chassis + wheels | 1 | Capstone project base |
| Tactile push button | 1 | Digital input |

## Software (All Free)

- Arduino IDE 2.x — arduino.cc/en/software

- Google Chrome — for optional online simulator (tinkercad.com)

# 5. Course Schedule at a Glance

Four modules, 2 hours each, 8 hours total. Can be delivered as a single day intensive or across multiple sessions.

| Mod | Title | Key Activities | Duration |
| --- | --- | --- | --- |
| 1 | Introduction to Robotics & Arduino | Robot anatomy, Arduino setup, Blink sketch, button input | 2 hours |
| 2 | Sensing the Environment | HC-SR04 ultrasonic sensor, LDR smart nightlight | 2 hours |
| 3 | Actuators & Motion Control | Servo motor, DC motor with L298N, proximity stop | 2 hours |
| 4 | Robot Decision Logic & Capstone | Obstacle-avoiding robot, 3 capstone challenge tracks | 2 hours |
| TOTAL COURSE DURATION |  |  | 8 hours |

Delivery Options Single Day Intensive: 4 sessions × 2 hours with 15-minute breaks between each. Two Half-Days: Modules 1–2 on Day 1, Modules 3–4 on Day 2. Weekly Sessions: One module per week over four weeks.

# 6. Module Descriptions

## Module 1 — Introduction to Robotics & Arduino

Getting familiar with what a robot is and making Arduino work for the first time.

| Time | Session | Type | Content |
| --- | --- | --- | --- |
| 0:00–0:45 | Robotics Fundamentals | Theory | What is a robot? Sense-Think-Act loop. Real-world robot examples. Arduino board anatomy — pins, power, USB. The microcontroller concept. |
| 0:45–1:20 | First Program — Blink | Lab | Install Arduino IDE. Upload the Blink sketch. Modify delay() values. Serial Monitor introduction. Debug first errors. |
| 1:20–2:00 | Button Input | Lab | Wire a tactile button with pull-down resistor. digitalRead() and if-else logic. LED responds to button press. Serial Monitor feedback. |

## Module 2 — Sensing the Environment

Connecting sensors and reading real-world data with the Serial Monitor.

| Time | Session | Type | Content |
| --- | --- | --- | --- |
| 0:00–0:40 | Sensor Theory | Theory | Types of sensors. Analog vs digital signals. Serial Monitor as a diagnostic tool. Voltage divider concept. |
| 0:40–1:20 | Ultrasonic Distance | Lab | HC-SR04 wiring. pulseIn() and distance formula. Alert LED when object within 15 cm. |
| 1:20–2:00 | LDR Smart Nightlight | Lab + Project | Voltage divider for LDR. analogRead(). Threshold logic. Build a working auto nightlight. |

## Module 3 — Actuators & Motion Control

Controlling motors and building movement into the robot.

| Time | Session | Type | Content |
| --- | --- | --- | --- |
| 0:00–0:35 | Motor Theory | Theory | DC motors vs servo motors. PWM explained. H-Bridge and why motor drivers exist. Current limitations of Arduino pins. |
| 0:35–1:20 | Servo Motor Control | Lab | SG90 wiring. Servo library. Sweep from 0–180. Potentiometer maps to angle via map(). |
| 1:20–2:00 | DC Motor + L298N | Lab + Project | L298N wiring. forward(), reverse(), stopMotor(). Proximity-stop mini-project with HC-SR04. |

## Module 4 — Robot Decision Logic & Capstone

Bring all skills together in a fully autonomous robot and live capstone demo.

| Time | Session | Type | Content |
| --- | --- | --- | --- |
| 0:00–0:30 | Decision Logic Theory | Theory | If-else as robot rules. Writing rules in plain language before code. Multi-condition logic. Serial debugging strategies. |
| 0:30–1:20 | Obstacle-Avoiding Robot | Lab | Assemble full robot. Wire HC-SR04 + L298N + 2 motors. Program stop, turn, forward logic. Floor testing. |
| 1:20–1:50 | Capstone Challenge | Project | Choose Track A/B/C. Build and code the enhancement. Peer testing. |
| 1:50–2:00 | Demo & Close | Demo | Live demo to class. Instructor scores rubric. Next steps discussion. |

# 7. Assessment & Grading

| Component | Weight | Description |
| --- | --- | --- |
| Lab Completion | 40% | Checklists completed in all 4 lab manuals |
| Mini-Projects | 30% | Smart nightlight (Mod 2), Proximity stop (Mod 3) — 15% each |
| Capstone Demo | 20% | Live robot demo scored on 100-point rubric |
| Participation | 10% | Engagement, peer support, asking questions |

# 8. Learning Outcomes

## Knowledge

- Define a robot and its three functional components

- Explain the Sense-Think-Act model with a real example

- Describe how ultrasonic distance measurement works

- Explain PWM and how it controls motor speed

## Skills

- Install Arduino IDE, upload sketches, and use Serial Monitor

- Wire sensors and actuators on a breadboard without soldering

- Write Arduino code with variables, conditionals, loops, and custom functions

- Build and test a fully autonomous obstacle-avoiding robot

## Attitudes

- Approach hardware and software problems with systematic debugging

- Treat errors as learning opportunities rather than failures

- Collaborate effectively with a lab partner

# 9. Policies & Resources

## Safe Use of Electronics

- Always power off the circuit before rewiring components

- Never connect a motor directly to Arduino pins — always use a driver

- Report any burning smell or heat to the instructor immediately

- Handle boards by their edges — static can damage components

## Recommended Resources

- arduino.cc/education — Official tutorials and projects

- tinkercad.com/circuits — Free browser-based Arduino simulator

- hackster.io — Step-by-step project guides and community builds

- youtube.com — Search 'Arduino for beginners' for video walkthroughs

A Note to Students In 8 hours you will go from knowing nothing about robotics to having built a real, working autonomous robot with your own hands. That is something to be proud of. Keep building — the skills you learn here open doors to robotics, AI, engineering, and computer science careers.